Chemical Safety

PURPOSE:  To prevent foodborne illness and employee injury due to chemical use.

SCOPE:  This procedure applies to all employees.

INSTRUCTIONS/PROCEDURES:

- Train all employees on chemical safety annually.

- Follow Island County Food Code requirements.

- Designate a location for Safety Data Sheets (SDS) that is accessible to all employees.

- Follow manufacturer’s directions for specific mixing, storing, and first aid instructions on the chemical containers in the SDS.

- Label all chemicals not stored in their original containers with the common name of the substance.

- Store all chemicals in a designated secured area away from food and food contact surfaces using spacing or partitioning.

- If possible, limit access to chemicals by use of locks, seals, or key cards.

- Maintain a master chemical log of all approved chemicals.

- Store only chemicals that are necessary to the operation and maintenance of the kitchen.

- Mix, test, and use sanitizing solutions as recommended by the manufacturer.

- Use the appropriate chemical test kit to measure the concentration of sanitizer each time a new batch of sanitizer is mixed.

- Do not use chemical containers for storing food or water.

- Use only hand sanitizers that comply with FDA regulations. Confirm with the manufacturer that the hand sanitizers used meet the requirements of FDA regulations.

- Label and store first aid supplies in an area that is located away from food or food contact surfaces.

- Label and store medicines and other personal care products for employee use in a designated area and away from food contact surfaces.  Do not store medicines in food storage areas.

- Store refrigerated medicines in a covered, leak proof container where they are not accessible to children and cannot contaminate food.

MONITORING:

- All employees will visually observe that chemicals are being stored, labeled, and used properly during all hours of operation.

CORRECTIVE ACTION:

- Retrain any employee who does not follow proper chemical safety procedures.

- Discard any food contaminated by chemicals.

- Label and properly store any unlabeled or misplaced chemicals.

VERIFICATION AND RECORD KEEPING:

- Management staff will audit compliance monthly.